UDAAN ROCKET SUBSYTEM INDUCTION TASK- 2026

SECTION 1: MECHANICAL AND FUEL CHEMISTRY TASK

TASK 1: Open Rocket (software and flight simulations)

Installation link: https://openrocket.info/

Reference link: https://www.youtube.com/live/xPFJwDYGqGE?si=1m_pK1wTblK2C1KL

Also go through these videos for reference:

• Basics of model rocketry by UDAAN NITR lecture: https://youtu.be/-0vGJchRWZg

• Rocket Stability: https://youtu.be/qCzF9OfYahc

Design a model rocket in open rocket following these specs strictly:

- Length <= 1000 mm

- Diameter <= 140 mm

- Desired stability margin = 1.5 cal

- Distance between CP & CG = 1.5 D (Dia of rocket)

- Use of coupler tube allowed

- Add proper recovery system and attain decent velocity < 6 m/s in simulation

- Body tube material is based on your choice and research

- Use any COTS motor available in software (preferred H to L class motor)

TASK 2: Openmotor (Rocket motor simulator)

Installation link : https://github.com/reilleya/openMotor/releases/tag/v0.5.0

{follow the above link go the bottom last ‘assets’ section, you will find link for windows and mac}

Open motor tutorial 1: https://youtu.be/jygAOqcnYVs?si=ijy-d85Ccq_gR37n

Open motor tutorial 2: https://youtu.be/YGUuGX2ru8c?si=g-NP34zsPyvLqjMB

Open motor tutorial 3: https://youtu.be/OmjEgMdfNeA?si=wZtGSks8ktabuUlU

Basics of rocket propulsion: https://youtu.be/SNpGBVoKOmg?si=Qy5t24fOnhGJPRRf

Rocket Motor codes explained: https://youtu.be/5Che6LOL0h0?si=vj5M9VNO077_gug1

Design and analyse 3 solid rocket motors using Open Motor with the following constraints:

- Propellant Type: KNSU or KNDX

- Grain geometry: BATES

- BATES Grain Diameter: 35 mm or 45 mm

- Maximum Thrust Configurations: 100 N, 200 N, 300 N

- Core diameter may be varied

- Grain length may be varied

- Effects on thrust curve, burn time, and total impulse must be considered

- Nozzle Optimization: convergent-divergent nozzle preferred, Throat diameter, Expansion ratio, Efficiency and performance trade-offs

SECTION 2: PRESENTATION TASK

TASK 3

Select Any 2 Topic from Link Given Below study them properly and Make a Detailed Presentation that you have to present during PI round. Sources from Nakka Rocketry :

https://www.nakka-rocketry.net/fins.html

https://www.nakka-rocketry.net/fusestru.html

https://www.nakka-rocketry.net/rocket.html

https://www.nakka-rocketry.net/burnrate.html

https://www.nakka-rocketry.net/sucrose.html

https://www.nakka-rocketry.net/sorb.html

https://www.nakka-rocketry.net/th_intro.html

https://www.nakka-rocketry.net/th_comb.html

https://www.nakka-rocketry.net/th_nozz.html

https://www.nakka-rocketry.net/th_thrst.html

https://www.nakka-rocketry.net/paracon.html

https://www.nakka-rocketry.net/pvcmot5.html

https://www.nakka-rocketry.net/pvcmot8.html

https://www.nakka-rocketry.net/therm.html

https://www.nakka-rocketry.net/

SECTION 3: CASE STUDY TASK

TASK 4

Choose one case study topic from below and make a informative ppt on that topic:

- Static test of KALAM 1200 solid motor at SDSC SHAR by Skyroot Aerospace.

- Boosters overview of starship (33 raptor engine) by space X

- Vikas engine- India

- ISRO HS200 solid rocket booster static test

- Vikram – S rocket by Skyroot Aerospace.

SECTION 4: ELECTRONICS TASK (BONUS TASK)

(not compulsory for all to attempt)

TASK 5

Design and simulate a circuit in Tinkercad for Arduino based Ignition circuit:

- Which ignites the rocket motor just after an interval of 10 seconds

- The timer is shown using a 7-segment display

- The time should run in reverse order

- Each second must be followed by a beep sound using buzzer

- Output Current should be greater than 1 Amp that should ignite the rocket ignitor

- Type a suitable code for above desired conditions

Resources and References:

● Arduino IDE: https://www.arduino.cc/en/software

● TinkerCad: https://www.tinkercad.com

● YouTube Playlist: https://youtube.com/playlist?list=PL4an6ELdlijducM1Y3oltzyFagC6Hx23Z

Prerequisites (Read about them in detail before proceeding with task):

1. Microcontrollers

● Arduino Nano

● Arduino Mega

● ESP32

2. Sensors & Modules

● BMP Sensor (Barometric Pressure Sensor)

● IMU Sensor

● GPS Module

3. Power & Control Electronics

● Voltage Regulator

● Buck Converter

● MOSFET

4. Communication

● Arduino Communication Protocols

○ UART

○ I2C

○ SPI